The Toughest Multimeters

FEATURES :

APPA 103N; APPA 105N; APPA 106; APPA 107; APPA 109. Main Display; 4000 Count; 4000 Count; 4000 Count; 20000/4000 Count; 20000/4000 Count. Dual Digital Display; NA; NA; NA. Analog Bar graph Display; 82 segments; 82 segments; 82 segments; 83 segments; 83 segments. Zoom Analog Bar graph; NA; NA; NA. Center Zero Analog Bar graph; NA; NA; NA. Self-Calibration; NA; NA; NA. True RMS; NA; NA; True RMS; AC+DC True RMS; AC+DC True RMS. Data Hold; NA; NA. Auto Hold; NA; NA; NA. Relative Mode; NA. Min/Max/Avg Function; NA. Peak Hold; NA. Store, Recall Function; NA; NA; NA; Up to 1000 records; Up to 1000 records. oC/oF Temperature Test; NA; NA. Auto Fuse Detector; NA; NA; NA. Beep Guard; NA; NA; NA. RS - 232 Cable; OPTION; OPTION; OPTION; OPTION; INCLUDED. WinDMM 100J software; OPTION; OPTION; OPTION; NA; NA. WinDMM 100 software; NA; NA; NA; OPTION; INCLUDED. Data logger records; NA; NA; NA; 6,000 records; 40000 records. dBm/ dB measurement; NA; NA; NA. Continuity Check/Beeper. Low Voltage Ohms; NA; NA; NA. Backlight. Open Communication Protocol. Water / Dust Resistant. High Energy Fuses. Protective Holster.

SPECIFICATIONS: (All at 23oC 5oC, < 80% R.H.)

APPA 103N; APPA 105N; APPA 106; APPA 107; APPA 109. DCV:; Ranges; 400mV ~ 1000V; 400mV ~ 1000V; 400mV ~ 1000V; 20mV ~ 1000V; 20mV ~ 1000V. Resolution; 0.1mV; 0.1mV; 0.1mV; 0.001mV; 0.001mV. Basic Accuracy; (0.25%+1d); (0.1%+1d); (0.1%+1d); (0.06%+10d); (0.06%+10d). ACV:; Ranges; 400mV ~ 750V; 400mV ~ 750V; 400mV ~ 750V; 20mV ~ 750V; 20mV ~ 750V. Resolution; 0.1mV; 0.1mV; 0.1mV; 0.001mV; 0.001mV. Basic Accuracy; (1.3%+ 5d); (1.0%+ 5d); (1.0%+ 5d); (0.70% + 50d), 40Hz ~ 100Hz; (0.70% + 50d), 40Hz ~ 100Hz. Bandwidth; 40Hz ~ 1KHz; 40Hz ~ 1KHz; 40Hz ~ 1KHz; 40Hz ~ 50KHz; 40Hz ~ 100KHz. Ranges; 400 ~ 40M; 400 ~ 40M; 400 ~ 40M; 200 ~ 2G; 200 ~ 2G. Resolution; 0.1; 0.1; 0.1; 0.01; 0.01. Basic Accuracy; (0.6%+ 2d); (0.4%+ 2d); (0.4%+ 2d); (0.30% + 30d), 200 ~ 200K; (0.30% + 30d), 200 ~ 200K. DIODE TEST:; Test Current; 1.5mA (Typical); 1.5mA (Typical); 1.5mA (Typical); 1.1mA (Typical); 1.1mA (Typical). Open circuit voltage; 3V DC (max).; 3V DC (max).; 3V DC (max).; 3.3V DC (max).; 3.3V DC (max).. Continuity Test:; Threshold; Approx.30; Approx.30; Approx.30; Approx.50; Approx.50. Indicator; 2KHz Tone Buzzer; 2KHz Tone Buzzer; 2KHz Tone Buzzer; 2KHz Tone Buzzer; 2KHz Tone Buzzer. DCA:; Ranges; 40mA ~ 10A; 40mA ~ 10A; 40mA ~ 10A; 20mA ~ 10A; 20mA ~ 10A. Resolution; 10A; 10A; 10A; 0.001mA; 0.001mA. Basic Accuracy; (0.6%+ 2d); (0.4%+ 2d); (0.4%+ 2d); (0.20%+ 40d); (0.20%+ 40d). ACA:; Ranges; 40mA ~ 10A; 40mA ~ 10A; 40mA ~ 10A; 20mA ~ 10A; 20mA ~ 10A. Resolution; 10A; 10A; 10A; 0.001mA; 0.001mA. Basic Accuracy; (1.3%+ 5d); (1.3%+ 5d); (1.3%+ 5d); (0.80%+ 50d), 40Hz ~ 500Hz; (0.80%+ 50d), 40Hz ~ 500Hz. Bandwidth; 40Hz ~ 1KHz; 40Hz ~ 1KHz; 40Hz ~ 1KHz; 40Hz ~ 3KHz; 40Hz ~ 3KHz. Capacitance:; Ranges; 4nF ~ 40mF; 4nF ~ 40mF; 4nF ~ 40mF; 4nF ~ 40mF; 4nF ~ 40mF. Resolution; 1pF; 1pF; 1pF; 1pF; 1pF. Basic Accuracy; (2%+ 5d); (2%+ 5d); (2%+ 5d); (0.90%+ 5d), 400nF ~ 4F; (0.90%+ 5d), 400nF ~ 4F. Hz:; Ranges; 4KHz ~ 40MHz; 4KHz ~ 400MHz; 4KHz ~ 400MHz; 20Hz ~ 1MHz; 20Hz ~ 1MHz. Resolution; 1Hz; 1Hz; 1Hz; 0.01Hz; 0.01Hz. Basic Accuracy; (0.01%+ 1d); (0.01%+ 1d); (0.01%+ 1d); (0.01%+ 10d); (0.01%+ 10d). RPM:; Ranges; 40KRPM ~ 400KRPM; 40KRPM ~ 400MRPM; 40KRPM ~ 400MRPM; NA; NA. Resolution; 30RPM; 30RPM; 30RPM. Basic Accuracy; (0.01%+ 10d); (0.01%+ 10d); (0.01%+ 10d). %DF:; Ranges; NA; NA; NA; 20% ~ 80%; 20% ~ 80%. Resolution; 0.1%; 0.1%. Basic Accuracy; 1% ( 20Hz ~ 10KHz, 5Vp-p ).; 1% ( 20Hz ~ 10KHz, 5Vp-p ).. Temperature:; Ranges; NA; NA; -20oC ~ 800oC; -200oC ~ 1200oC; -200oC ~ 1200oC. Resolution; 1oC; 0.1 oC; 0.1 oC. Basic Accuracy; (2%+ 4oC), -20oC ~ 350oC (2%+ 2oC), 350oC ~ 500oC (3%+ 2oC), 501oC ~ 800oC; (0.1%+6oC), -200oC ~ 100oC (0.1%+3oC), 100oC ~ 1200oC; (0.1%+6oC), -200oC ~ 100oC (0.1%+3oCs), 100oC ~ 1200oC.

GENERAL SPECIFICATIONS:

APPA 103N; APPA 105N; APPA 106; APPA 107; APPA 109. Sampling Rate: 2.5times/sec. Auto Power Off: 30 minutes. Operating Temperature: 0oC ~ 50oC. Storage Temperature: -20oC ~ 60oC 0 to 80% R.H.. Temperature: Coefficient: 0.15x(Spec.Sccuracy)/oC, <18oC, >28oC. Safty Standard: IEC 61010 & designed to meet UL3111 specifications.; CAT. III 600V, CAT. II 1000V.; CAT. III 600V, CAT. II 1000V.; CAT. III 600V, CAT. II 1000V.; CAT. III 600V, CAT. II 1000V.; CAT. III 600V, CAT. II 1000V. Battery Life: in hours ( Alkaline battery ); 450; 450; 300; 100; 100. Power Requirement: Single 9V battery (NEDA 1604 or IEC 6F22 ); NA; NA. IEC, LR03, AM4 Aor AAA Size 1.5 x2; NA; NA; NA. Dimensions: 86mm (W) x 185mm (L) x 31.5mm (D) 98mm ( W ) x 197mm ( L ) x 50mm ( D ) with holster. Weight: in grams (with holster); 620; 620; 620; 620; 620. Accessories:; Protective holster, test leads, test clips, ZnC battery (installed) and manual.; Protective holster, test leads, test clips, ZnC battery (installed) and manual.; Protective holster, test leads, test clips, Temp. adaptor, Bead sensor, Alkaline, battery (installed) and manual.; Protective holster, test leads, test clips, Temp. adaptor, Bead sensor. Alkaline battery (installed) and manual.; Protective holster, test leads, test clips, Temp. adaptor, Bead sensor. Alkaline battery (installed) and manual.. Optical RS 232 Cable:; OPTION; OPTION; OPTION; OPTION; INCLUDED. APPA WinDMM 100J software:; OPTION; OPTION; OPTION; NA; NA. APPA WinDMM 100 software:; NA; NA; NA; OPTION; INCLUDED.